# QUIET SUM

Description :-

Relay station KX-7 went dark three weeks ago. Its last transmission was one RSA-encrypted message, and the operators were careful: the private key never left the hardware security module.

The firmware, however, liked to talk. Before every key-generation cycle the HSM broadcast a "health check" built from an internal register, and someone left the debug filter on, so each broadcast picked up a little jitter. We captured five of those readings, plus the public key and the ciphertext.

The register was never meant to be secret. It was never meant to be heard either.

Flag format: VOID{...}

Files :-

Instance.json

{

"modulus": "4973157204747533894108763601161346203976511386649435211597325896047088474094725330801808127345046312007232066738067902013021045088941098964150392081899091",

"exponent": 65537,

"encrypted": "4334473244409174013915666087363991504894518568914364005011530030087290179357390032679558601071125432766129376181647994322533385035132103114260099689097875",

"telemetry": [

"543822096420199366166609185931411161676415981992005522084052043092032713",

"543822096420199366166609185931411161676415981992005522084052043092032716",

"543822096420199366166609185931411161676415981992005522084052043092032716",

"543822096420199366166609185931411161676415981992005522084052043092032716",

"543822096420199366166609185931411161676415981992005522084052043092032714"

]

}

Approach :-

We are given a public modulus N, the exponent 65537 & a ciphertext so this is clearly an RSA challenge. Normally to read the message we have to factor N = p × q & rebuild the private key but since N is 511 bits which is too big to factor directly so the attack has to come from the telemetry.

The five readings are almost the same number & differ by only a few units so they are all noisy copies of one hidden value. When I compared its size with the size of the RSA values, N is 511 bits so p & q are about 256 bits each & their sum p + q is about 257 bits. The readings are only 239 bits long which is 18 bits less, which suggests the readings are the upper bits of p + q with its lowest 18 bits cut off & the small differences between them are noise on top. Hence this is a partial key exposure situation which is also pretty common in CTFs, where a part of the secret key material is leaked & is used to factor N.

On searching online about the types of attacks I can use to recover the message, I came across some partial key exposure attacks on RSA which come from side-channel attacks where only some bits of the secret are known. Most of them need lattice reduction to recover the missing bits but here only 18 bits are missing, which is just 2^18 = 262144 possibilities & can be brute forced so the plan was to take the upper bits of p + q from the readings & then try every value of the 18 missing bits to find the one that lets us factor N & then decrypt the ciphertext in the normal RSA way.

Solution :-

First we will start by looking at the telemetry & comparing it with the RSA values so we write a small script using python :-

import json

with open("instance.json") as f:

data = json.load(f)

n = int(data["modulus"])

readings = [int(x) for x in data["telemetry"]]

print(n.bit_length(), readings[0].bit_length())

print([r - min(readings) for r in readings])

Output :-

511 239

[0, 3, 3, 3, 1]

So each reading looks like S >> 18 which means we know the upper bits of S & only its lowest 18 bits are missing & knowing S would be enough to factor N without any hard work since N = p·q, p & q are the roots of the quadratic :-

x² - S·x + N = 0

Its discriminant is :-

D = S² - 4N = (p - q)²

So for the correct S, D has to be a perfect square, & then :-

p = (S + √D) / 2

q = (S - √D) / 2

This gives us a test for every guess of S. Only the lowest 18 bits are unknown so there are 2^18 guesses for each reading & since p & q are odd primes so S is always even & we only need to test the even values which is 2^17 = 131072 guesses. We don’t know which reading holds the exact upper bits because of the jitter so we try every value between the smallest & the largest reading so we write a small script to run this search using python :-

import json

from math import isqrt

with open("instance.json") as f:

data = json.load(f)

n = int(data["modulus"])

readings = [int(x) for x in data["telemetry"]]

for t in range(min(readings), max(readings) + 1):

for low in range(0, 1 << 18, 2):

s = (t << 18) + low

d = s * s - 4 * n

if d < 0:

continue

r = isqrt(d)

if r * r == d:

print("p =", (s + r) // 2)

print("q =", (s - r) // 2)

raise SystemExit

Output :-

p = 81655773197653871906320441565853116073813936807306122135322362555338716611047

q = 60903926446322870738059156870950731492688454376006173445879376228979106908853

The search gives us two numbers so this is almost certainly the real factorization. Now with p & q we can rebuild the private key as φ(N) = (p - 1)(q - 1), d is the inverse of e modulo φ(N) & the message is m = c^d mod N so we write a small script to get the flag using python :-

import json

with open("instance.json") as f:

data = json.load(f)

n = int(data["modulus"])

e = int(data["exponent"])

c = int(data["encrypted"])

p = 81655773197653871906320441565853116073813936807306122135322362555338716611047

q = 60903926446322870738059156870950731492688454376006173445879376228979106908853

d = pow(e, -1, (p - 1) * (q - 1))

m = pow(c, d, n)

print(m.to_bytes((m.bit_length() + 7) // 8, "big").decode())

Output :-

VOID{quadratic_discriminants_reveal_the_quiet_sum}

Flag :-

VOID{quadratic_discriminants_reveal_the_quiet_sum}